SM-GE3222M Smart Energy Monitor
V2.0 Firmware & VB.NET Integration Scope

Project Scope and Architecture Definition (Pre-Implementation)
Date: 20 Feb 2026

# 1. Purpose and Scope

This document defines the complete end-to-end scope for the ESP32-based SM-GE3222M Energy Meter V2.0 firmware and its VB.NET application integration. It is intended to be finalized before coding begins, ensuring consistent architecture, interfaces, register maps, and validation/diagnostics behavior.

Firmware target: ESP32 (Arduino framework, FreeRTOS tasks).

Energy IC driver: EnergyATM90E36.h/.cpp MUST remain unchanged from V1.0.

Hardware configuration and calibration defaults MUST match V1.0 Definitions.h baseline.

Modbus TCP is removed completely. Modbus RTU only.

Ethernet TCP/IP is used for general TCP JSON communications today; future BACnet/IP will reuse Ethernet stack.

Web UI is optional but supported; it must consume the same runtime data model as Modbus and TCP.

# 2. Key Design Constraints

Do not modify EnergyATM90E36.h/.cpp (bit-identical to V1.0).

SPI pins for ATM90E36: CS=GPIO5, SCK=GPIO18, MISO=GPIO19, MOSI=GPIO23.

MCP23017 I2C address 0x20 with interrupt outputs: INT_A=GPIO25, INT_B=GPIO26.

DHT22 sensor input: DHTPIN=GPIO4.

DC voltage sensing (ADC1): DCV_IN=GPIO36.

LCD: 20x4 LiquidCrystal_I2C address 0x27.

Stable networking priority: Ethernet -> WiFi STA-only -> AP-only fallback (no mode flapping).

Single source of truth runtime data model shared across LCD, Web, TCP JSON, and Modbus RTU.

# 3. Hardware Configuration

All pins and peripherals below are authoritative for V2.0 and must match the V1.0 baseline where applicable.

| Subsystem | Device / Interface | Pins / Address | Notes / Ownership |
| --- | --- | --- | --- |
| MCU | ESP32 | - | Arduino framework, FreeRTOS tasks |
| Energy IC | ATM90E36 (SPI) | CS=5, SCK=18, MISO=19, MOSI=23 | SPIBus initializes pins; driver unchanged |
| I/O Expander | MCP23017 (I2C) | Addr 0x20; INT_A=25; INT_B=26 | All expander IO via MCP23017Hal |
| LCD | LiquidCrystal_I2C | Addr 0x27; 20x4 | LCDManager uses shared I2C bus |
| Sensor | DHT22 | GPIO4 | DHT22Service only; timed reads (>=2s) |
| Analog Sense | DC Voltage ADC | GPIO36 (ADC1) | DcVoltageService + AdcHal; filtered & scaled |
| Buttons | Mode, Set | GPIO32, GPIO33 | Debounced; UI events |
| Ethernet | W5500 (SPI or dedicated) | per board | General TCP/IP today; future BACnet/IP |
| Wi-Fi | ESP32 radio | - | STA-only or AP-only; no dual mode |

MCP23017 logical channel assignment (example as per V1 baseline):

| MCP Pin Index | Signal | Direction / Use |
| --- | --- | --- |
| 0 | RELAY_2 | Output |
| 1 | RELAY_1 | Output |
| 2 | WIFI_LED | Output |
| 3 | RUN_LED | Output |
| 4 | FLT_LED | Output |
| 5 | MODBUS_LED | Output |
| 11 | CF4 | Input (pulse) |
| 12 | CF3 | Input (pulse) |
| 13 | CF2 | Input (pulse) |
| 14 | CF1 | Input (pulse) |

# 4. Proposed Firmware Directory Structure

The V2.0 project remains within the existing Arduino sketch structure and adds modular code under src/.

SM_GE3222M/
  SM_GE3222M.ino                    (thin: boot + scheduler start)
  Definitions.h                     (V1 baseline: calibration + hardware constants)
  EnergyATM90E36.h/.cpp             (UNTOUCHED driver)
  data/                             (SPIFFS web assets)
  include/
    PinMap.h
    GlobalTypes.h
    Version.h
    ModbusRegMap.h                  (constants only)
  src/
    core/
      SystemManager.h/.cpp          (boot phases, SAFE MODE, orchestrator)
      TaskScheduler.h/.cpp          (cooperative tick scheduler)
      TimerManager.h/.cpp
      EventBus.h/.cpp               (optional)
    hal/
      BoardConfig.h/.cpp            (authoritative pins/feature flags)
      SPIBus.h/.cpp                 (SPI pin init: 18/19/23 + CS5)
      I2CBus.h/.cpp                 (shared Wire + mutex)
      MCP23017Hal.h/.cpp            (IO + interrupt capture)
      AdcHal.h/.cpp                 (ADC1 reads + filtering helpers)
      GPIOManager.h/.cpp            (buttons, esp gpio)
    energy/
      EnergyMeter.h/.cpp            (wraps unchanged driver; validates readings)
      Calibration.h/.cpp            (apply + store validation)
      EnergyAccumulator.h/.cpp      (persistent totals)
    sensors/
      DHT22Service.h/.cpp           (GPIO4)
      DcVoltageService.h/.cpp       (GPIO36)
    comm/
      EthernetManager.h/.cpp        (general TCP/IP + link status)
      WiFiManager.h/.cpp            (STA-only/AP-only; fallback policy)
      TcpJsonLinkService.h/.cpp     (ESP32 <-> VB TCP JSON)
      ModbusRTUServer.h/.cpp        (Modbus RTU only; no Modbus TCP)
      ModbusMap.h/.cpp              (getters/setters wired to SystemContext)
      WebServerManager.h/.cpp       (optional HTTP/WS)
      APIServer.h/.cpp              (JSON endpoints for Web)
    display/
      LCDManager.h/.cpp             (LiquidCrystal_I2C 0x27, 20x4)
      LEDIndicator.h/.cpp           (status patterns)
    storage/
      SPIFFSManager.h/.cpp          (mount + file helpers)
      ConfigManager.h/.cpp          (/config.txt compatible)
      NVStorage.h/.cpp              (optional NVS mirror)
      DataLogger.h/.cpp             (event log ring buffer)
    diagnostics/
      Logger.h/.cpp                 (ring buffer + levels)
      ErrorHandler.h/.cpp           (codes, severity, recovery policy)
      HealthMonitor.h/.cpp          (reboot loop detect, heap checks)
      WatchdogManager.h/.cpp        (task liveness + WDT feed)

# 5. Proposed V2.0 Firmware Architecture

Architecture is layered to prevent cross-coupling, eliminate blocking behavior, and ensure one data model feeds all interfaces.

+--------------------------------------------------------------+
|                         Application Layer                    |
|  LCD UI  |  Web UI (optional) | TCP JSON Link | Modbus RTU    |
+-------------------------+------------------------------------+
|                      Services Layer                           |
|  EnergyService | SensorServices | ConfigStore | Diagnostics    |
+-------------------------+------------------------------------+
|                    Hardware Abstraction Layer                 |
|  SPIBus | I2CBus | MCP23017Hal | AdcHal | GPIOManager         |
+-------------------------+------------------------------------+
|                 Platform / OS (ESP32 Arduino)                 |
|  FreeRTOS tasks, WDT, WiFi/Ethernet stacks, SPIFFS            |
+--------------------------------------------------------------+

Key rules:

Only HAL modules access hardware buses (SPI/I2C/ADC/GPIO).

Only service modules update the SystemContext runtime model.

UI and protocols are read-only consumers of SystemContext except via controlled command APIs.

No I2C or SPI operations inside ISRs; ISRs only set flags for tasks.

# 6. Firmware Data Flow Block Diagram

+--------------------+
                        |   ConfigManager    |
                        |  /config.txt + CRC |
                        +----------+---------+
                                   |
                                   v
+-------------+     +--------------+--------------+      +------------------+
| ATM90E36 IC | --> | EnergyMeter (driver wrapper) | --> |                  |
+-------------+     +--------------+--------------+      |                  |
                                   |                     |                  |
                                   v                     |                  |
+-------------+     +--------------+--------------+      |   SystemContext  |--> LCDManager
|   DHT22     | --> | DHT22Service (timed reads) | ----> |  (single truth)  |--> WebServer/APIServer
+-------------+     +--------------+--------------+      |                  |--> TcpJsonLinkService
                                   |                     |                  |--> ModbusRTUServer
                                   v                     |                  |
+-------------+     +--------------+--------------+      |                  |
| ADC GPIO36  | --> | DcVoltageService (filter)  | ----> |                  |
+-------------+     +--------------+--------------+      +------------------+
                                   ^
                                   |
                    +--------------+--------------+
                    | MCP23017Hal (INTA/INTB)     |
                    | pulse + DI capture + DO set |
                    +-----------------------------+

Data update policy: energy/sensors/network tasks write to SystemContext; protocol/UI tasks read only.

# 7. Communication Protocol V2 (TCP JSON, Backward Compatible)

TCP protocol is newline-delimited JSON frames. It remains backward compatible by preserving existing keys used by the VB.NET application, while adding optional V2 fields.

Transport: TCP socket over Ethernet or WiFi (STA/AP).

Framing: UTF-8 JSON per line (\n). No binary framing required.

Compatibility: V1 keys are preserved; V2 adds optional fields under 'v2' object where needed.

Reliability: sequence counters + acknowledgements for commands; reconnect safe.

Message types (minimum set):

| Type | Direction | Purpose | Required Fields |
| --- | --- | --- | --- |
| hello | ESP32 -> VB | Device identity and capabilities | fw, hw, mac, mode, features |
| status | ESP32 -> VB | Network + health snapshot | uptime, net, rssi, heap, errors |
| readings | ESP32 -> VB | Energy + sensor readings | seq, ts, energy{}, sensors{} |
| config | VB -> ESP32 / ESP32 -> VB | Get/set config snapshot | action=get¦set, payload{} |
| cmd | VB -> ESP32 | Command execution | id, name, args{} |
| ack | ESP32 -> VB | Acknowledgement for cmd/config | id, ok, code, msg |
| error | ESP32 -> VB | Asynchronous fault notification | code, severity, where |

## 7.1 TCP Protocol Sequence Diagram (ESP32 ↔ VB.NET)

VB.NET                                          ESP32
  |                                               |
  |--- TCP connect ------------------------------->|
  |<-- hello {fw,hw,mac,features} -----------------|
  |--- config {action:'get'} --------------------->|
  |<-- config {action:'snapshot',payload:{...}} ---|
  |                                               |
  |<-- status {net,heap,uptime,errors[]} ----------|  (periodic 1-5s)
  |<-- readings {seq,ts,energy{},sensors{}} -------|  (periodic publish interval)
  |                                               |
  |--- cmd {id:101,name:'setRelay',args:{...}} --->|
  |<-- ack {id:101,ok:true} ----------------------|
  |                                               |
  |--- cmd {id:102,name:'saveConfig'} ------------>|
  |<-- ack {id:102,ok:true} ----------------------|
  |                                               |
  |<-- error {code:'NET-002',severity:'WARN'} -----|  (async if needed)
  |                                               |
  |--- TCP close / reconnect --------------------->|

## 7.2 Multi-Client Architecture (Normal Operation)

+-------------------+
             |   VB.NET Client   |
             +---------+---------+
                       | TCP JSON
                       v
+----------------------+------------------------+
|            ESP32 Communication Layer          |
|  TcpJsonLinkService | ModbusRTUServer | WebUI |
+-----------+-------------------+---------------+
            |                   |
            v                   v
        SystemContext       SPIFFS/Web assets
            ^
            |
   Energy/Sensors/Network tasks

# 8. Configuration & Storage Architecture

Configuration storage is designed for safety (CRC + validation), backward compatibility (same filenames), and low risk of corruption.

Primary store: SPIFFS files (same filenames as V1): /config.txt, /Calibration_1.txt.. /Calibration_4.txt.

Each file includes a V2 header (version + CRC) while preserving V1 key names and layout.

Validation and defaulting: missing keys -> defaults; invalid keys -> revert default + raise CFG warning/error.

Optional NVS mirror (NVStorage) may cache validated config for quicker boot; SPIFFS remains authoritative for export/import.

SPIFFS
  /config.txt
    - device settings (network, ports, publish interval, feature enables)
    - V2 header: version, CRC32, timestamp
  /Calibration_1..4.txt
    - calibration blocks (register-style values)
    - V2 header: version, CRC32, timestamp
  /eventlog.bin (optional)
    - ring buffer of diagnostics/events
  /totals.bin (optional)
    - persistent energy totals + CRC

Write policy to reduce wear:

Only write on explicit user command (save config) or controlled periodic commit for totals (e.g., every 5-15 minutes).

Atomic update pattern: write temp file then rename to target to avoid partial writes.

CRC check on read; if CRC fails -> load defaults and set persistent error flag.

# 9. Error Handling & Diagnostics Architecture

V2 introduces structured error codes, severity levels, recovery policies, and observability across LCD, Modbus, and TCP JSON.

| Category | Examples | Severity | Recovery Policy |
| --- | --- | --- | --- |
| BOOT | boot timeout, reboot loop | FATAL/ERROR | enter SAFE MODE; limit services; require manual reset for some |
| CFG | missing key, CRC fail | WARN/ERROR | default values; persist error; continue |
| ENERGY | ATM90E36 init fail, invalid readings | ERROR/FATAL | retry with backoff; degrade mode; show error |
| NET | STA connect fail, ETH link down | WARN | fallback policy: ETH->STA->AP |
| I2C/SPI | bus timeout, MCP missing | ERROR | retry; isolate IO; continue with limited function |
| MODBUS | RTU framing errors, overload | WARN | throttle; increment counters |
| SENS | DHT timeout, ADC overrange | WARN | keep last valid; mark stale |

Error source -> ErrorHandler (code + severity + context)
           -> Logger (ring buffer)
           -> HealthMonitor (counters, reboot loop detect)
           -> Expose to:
              - LCD status page (human readable)
              - Modbus IR: last_error_code, boot_phase, counters
              - TCP JSON: async 'error' frames

# 10. FreeRTOS Task Architecture

Tasks are non-blocking and use a cooperative scheduler tick where possible; long operations are broken into state machines.

| Task | Owner Module | Priority | Period/Trigger | Core | Notes |
| --- | --- | --- | --- | --- | --- |
| BootTask | core/SystemManager | High | One-shot state machine | Any | Phased init with timeouts |
| MCPInterruptTask | hal/MCP23017Hal | High | INT_A/INT_B flags | Any | Reads INTCAP; updates DI/pulses |
| EnergyReadTask | energy/EnergyMeter | Med-High | 200-500 ms | Any | Updates MeterReadings |
| DHT22Task | sensors/DHT22Service | Low | 2 s | Any | Timed reads; no blocking loops |
| DCVTask | sensors/DcVoltageService | Low-Med | 200-500 ms | Any | ADC oversample + filter |
| NetworkTask | comm/*Manager | Med | State machine | Any | ETH->STA->AP fallback |
| TcpJsonTask | comm/TcpJsonLinkService | Med | Publish interval + socket events | Any | Frames + ACK |
| ModbusRTUTask | comm/ModbusRTUServer | Med | 10-50 ms tick | Any | RTU only |
| WebTask (optional) | comm/WebServerManager | Low-Med | event-driven | Any | HTTP endpoints |
| LCDTask | display/LCDManager | Low | 250-500 ms | Any | Draw diffs only |
| HealthTask | diagnostics/HealthMonitor | Low | 1-5 s | Any | Heap + reboot loop detect |
| WDTFeedTask | diagnostics/WatchdogManager | High | 100-250 ms | Any | Feeds only if tasks healthy |

# 11. Boot Sequence Diagram

PHASE 0  Reset reason + minimal logger
PHASE 1  SPIFFS mount + load config + load calibration (CRC validate)
PHASE 2  HAL init (SPI pins, I2C, ADC, GPIO, attach INT_A/INT_B)
PHASE 3  MCP23017 init + interrupt-on-change setup + clear captures
PHASE 4  LCD init (0x27, 20x4) + buttons + boot UI
PHASE 5  Energy IC init (unchanged driver) + retry/backoff + safe mode rules
PHASE 6  Sensors start (DHT22 + DCV)
PHASE 7  Network start (ETH -> STA -> AP fallback)
PHASE 8  Start services: TCP JSON, Modbus RTU, optional Web UI
RUN      Periodic read/update + diagnostics + watchdog

SAFE MODE entry conditions (examples):

Energy IC fails init after N retries OR repeated reboot loop detected.

SPIFFS mount fails or configuration CRC repeatedly invalid.

In SAFE MODE: allow AP-only + minimal web/config + diagnostics; disable high-rate sampling; keep Modbus RTU optional.

# 12. Modbus RTU Register Map Overview

Modbus TCP is removed. This map is for Modbus RTU using emelianov/modbus-esp8266. Encoding follows V1-style scaled int32 across 2 registers.

Float-style values: int32 scaled by 10 across two 16-bit registers (low word first).

uint32 counters: two registers (low word first).

Bitfields: uint16.

IPv4: two registers (A<<8|B, C<<8|D).

MAC: three registers.

| Block | Type | Range (hex) | Contents |
| --- | --- | --- | --- |
| Input Registers | 3x (RO) | 0x0000-0x009F | V1 energy + RMS/THD/angles (unchanged) |
| Input Registers | 3x (RO) | 0x00B0-0x00BF | Sensors: DHT22, DCV, statuses |
| Input Registers | 3x (RO) | 0x00C0-0x00CF | IO: relays/DI bitmaps, CF pulse counters |
| Input Registers | 3x (RO) | 0x00D0-0x00DF | Network runtime: mode, IPs, RSSI |
| Input Registers | 3x (RO) | 0x00E0-0x00EF | Diagnostics: uptime, heap, last error |
| Holding Registers | 4x (RW) | 0x0000-0x0048 | Calibration arrays (stride=2) |
| Holding Registers | 4x (RW) | 0x0080-0x0088 | Calibration config (MMode0/1 thresholds) |
| Holding Registers | 4x (RW) | 0x0200-0x0210 | Modbus RTU & COM settings |
| Holding Registers | 4x (RW) | 0x0300-0x0384 | Identity strings + MAC + board features |
| Holding Registers | 4x (RW) | 0x0400-0x040C | Ethernet config (DHCP/static, port) |
| Holding Registers | 4x (RW) | 0x0500-0x052C | WiFi STA config (SSID/pass + IP) |
| Holding Registers | 4x (RW) | 0x0600-0x0623 | WiFi AP config (SSID/pass + channel + IP) |
| Holding Registers | 4x (RW) | 0x0700-0x0704 | Web + TCP JSON link settings |
| Holding Registers | 4x (RW) | 0x0800-0x0849 | Logging, commands, event log window |
| Holding Registers | 4x (RW) | 0x0850-0x0854 | Pulse output settings (V1 compatible) |

Detailed constants are implemented in include/ModbusRegMap.h and consumed by src/comm/ModbusMap.* getters/setters.

# 13. Implementation Scope & Feature Matrix (V1.0 → V2.0)

| Area | V1.0 | V2.0 Target | Status in V2 Plan | Notes |
| --- | --- | --- | --- | --- |
| Energy IC driver | Working but tightly coupled | Driver unchanged; wrapper + validation | Must keep driver identical | EnergyATM90E36.* untouched |
| Calibration | SPIFFS text, limited validation | CRC + versioning + atomic writes | Planned | Backward compatible filenames |
| Networking | Unstable switching | ETH -> STA-only -> AP-only stable | Planned | No mode flapping |
| Web UI | Partial/unstable | Optional HTTP using shared model | Planned | No blocking in web task |
| VB comms | Mixed / evolving | TCP JSON V2 + backward keys | Planned | Sequence + ACK |
| Modbus | RTU + sometimes TCP | RTU only (TCP removed) | Mandatory | Uses emelianov/modbus-esp8266 |
| MCP23017 IO | Basic polling | Interrupt-driven capture + IO shadow | Planned | INT_A=25 INT_B=26 |
| Sensors | Not unified | DHT22 + DCV via services | Added | GPIO4 + GPIO36 |
| Diagnostics | Ad-hoc logs | Error codes + health monitor + event log | Added | Exposed via Modbus + TCP |
| WDT stability | WDT triggers observed | Non-blocking state machines + liveness checks | Key goal | WatchdogManager gates feed |

# 14. Implementation Roadmap

Roadmap is phased to deliver stability first, then features, while keeping the driver unchanged.

| Phase | Primary Deliverables | Validation / Tests | Exit Criteria |
| --- | --- | --- | --- |
| Phase A - Foundations | Directory structure, SystemContext, Logger, ErrorHandler, SPIFFS config loader (CRC) | Boot without reset; config load + defaulting; basic serial logs | Stable boot, no WDT, correct pin init |
| Phase B - Hardware HAL + IO | SPIBus pins fixed; I2CBus; MCP23017Hal + INT handling; GPIO + buttons | Interrupt fires; pulse counters increment; outputs controllable | No I2C lockups; stable ISR->task handoff |
| Phase C - Energy layer | EnergyMeter wrapper, retries/backoff, reading validation; accumulator | Energy readings update at rate; invalid reads flagged; totals persist | No boot halt; readings consistent with V1 |
| Phase D - Sensors | DHT22Service + DcVoltageService; add to model + Modbus | DHT reads every 2s without blocking; DCV filtered | Sensor statuses and values visible |
| Phase E - Comms | NetworkManager ETH->STA->AP; TCP JSON link; Modbus RTU map; optional Web | Reconnect tests; multi-client; command/ack tests; Modbus scan | VB stable connect; Modbus consistent; no resets |
| Phase F - Diagnostics hardening | HealthMonitor, WDT gating, event log; performance tuning | Soak test 24-72h; simulated network drops; heap stability | Meets metrics; no watchdog resets |

# 15. Improvement Metrics Summary

Quantitative targets to confirm V2.0 improvements:

| Metric | Baseline (V1 typical) | V2 Target | How to Measure |
| --- | --- | --- | --- |
| Boot time to stable services | Variable; may halt/reset | < 20 s to RUN state | Timestamp logs per boot phase |
| WDT resets | Observed in web/network tasks | 0 resets in 72h soak | Reset reason counter + event log |
| WiFi STA reconnect time | Unstable | < 10 s after AP/STA loss | NetworkTask timing + VB status frames |
| Ethernet link recovery | N/A/variable | < 5 s after link restore | ETH status + IP lease time |
| Heap minimum | Drops under load | > 40 KB minimum free heap | HealthMonitor min heap register |
| CPU load | High during polling | Stable under 60% peak | Profiler counters / loop timing |
| Energy read cadence | Jittery during comms | Consistent +/- 10% | Sequence timestamps |

# 16. End-to-End Communication Sequence Diagrams

## 16.1 Normal Operation (ETH active)

BOOT -> NetworkTask selects Ethernet -> TCP JSON server starts -> Modbus RTU ready
VB connects:
  1) hello + capabilities
  2) config snapshot (optional)
  3) periodic status + readings
Commands:
  - VB sends cmd (e.g., set relay / save config)
  - ESP32 validates -> applies -> ack
Modbus RTU:
  - PLC/HMI polls energy (IR 0x0000..0x009F) and sensors/net/diag blocks
Web UI (optional):
  - Browser reads /api/status and /api/readings (same SystemContext)

## 16.2 Multi-Client (VB + Web + Modbus RTU)

Clients:
  - VB.NET (TCP JSON)          (high-level control + config + graphs)
  - PLC/HMI (Modbus RTU)       (industrial integration)
  - Browser (HTTP) optional    (service UI)
All clients read the same SystemContext snapshot.
Write operations are serialized through command handlers:
  VB cmd/config -> CommandQueue -> SystemManager applies -> ack -> optional persist

# Appendix A. JSON Frame Examples (V2 compatible)

hello:
{"type":"hello","fw":"2.0.0","hw":"GE3222M-A","mac":"C8:2E:A3:F5:7D:DC","features":{"eth":1,"wifi":1,"dht22":1,"dcv":1,"lcd":1,"mcp":1}}

readings:
{"type":"readings","seq":1234,"ts":1739960000,
 "energy":{"uA":2305,"uB":2310,"uC":2298,"iA":120,"iB":110,"iC":98,"pT":3520},
 "sensors":{"tC":285,"rh":520,"dcv":123}
}

cmd:
{"type":"cmd","id":101,"name":"setRelay","args":{"ch":1,"on":true}}

ack:
{"type":"ack","id":101,"ok":true}

# Appendix B. Modbus Blocks Summary

Use include/ModbusRegMap.h for full constants. Key blocks:

IR 0x0000-0x009F: V1 energy block (unchanged).

IR 0x00B0-0x00BF: sensors (DHT22, DCV).

IR 0x00C0-0x00CF: IO (bitmaps + CF pulse counters).

IR 0x00D0-0x00DF: network runtime.

IR 0x00E0-0x00EF: diagnostics.

HR blocks: calibration (0x0000+), config, identity, network settings, logging, commands.

End of document.